Coursework

Line Follower Robot

6COM2009 Real-Time and Concurrent Systems (RTCS) 2025

Initio Robot Car – ISO C Implementation

Hein Pyae Sone Phyo

# Section 1: I/O Control

In this coursework I programmed the Initio robot car using ISO C to control its infrared sensors and DC motors. The robot platform provides functions through the initio library, allowing sensor reading and actuator control using PWM.

Sensors Used:

The robot uses two types of IR sensors:

- Front Obstacle IR Sensors 	Functions used:

- initio_IrLeft()

- initio_IrRight()

These return an integer indicating whether an obstacle is detected in front of the car. A non-zero return value means an object is present. These values are used to trigger automatic stopping.

- Line-Following IR Sensors (pointing at floor)

Functions used:

- initio_IrLineLeft()

- initio_IrLineRight()

These detect the contrast between the black line and the white floor. The returned values are checked to determine whether each sensor is over the line or not.

Actuators Used:

The robot uses two DC motors for movement. The following functions were used:

- initio_DriveForward(int speed)

Drives both motors forward at a PWM value between 0–100.

- initio_SpinLeft(int speed)

Spins the car left by running the motors in opposite directions.

- initio_SpinRight(int speed) Spins the car right.

PWM controls the energy delivered to the DC motors, affecting speed. Higher PWM increases speed, and friction affects turning performance. initio_Init() and initio_Cleanup() handle setup and shut-down.

# Section 2: Obstacle Avoidance

The robot uses initio_IrLeft() and initio_IrRight() to detect obstacles. If either sensor detects an object, the robot executes initio_DriveForward(0) to stop. When both sensors return zero again (object gone), the robot automatically resumes the line-following.

Fault Tolerance: The use of two obstacle sensors provides redundancy. If one sensor temporarily misreads or becomes misaligned, the second sensor still prevents the robot from driving into an object. The logic also continuously checks for obstacles, so the car will not resume driving until both sensors report a clear state.This ensures a simple but effective level of safety and fault tolerance.

# Section 3: Line Follower

The line follower uses initio_IrLineLeft() and initio_IrLineRight(). The control logic is:

- Both BLACK: drive forward

- Left BLACK, Right WHITE: spin left

- Left WHITE, Right BLACK: spin right

- Both WHITE: line lost: recovery strategy

Forward speed was set to 87 and turning speed to 100. The binary nature of the sensors limits the use of continuous control methods like PID.

## Refinement of Line Follower Algorithm

A recovery mechanism was added using lastDirection. If the line is lost, the robot spins in the last known turning direction to re-acquire the line. This improves performance on curves and noisy line segments

# Section 4: Implementation Details

The implementation is contained in a single C source file and a Makefile:

- line_follower.c – sensor reading, decision logic, motor control, curses status display, and program initialisation/cleanup.

- Makefile – compilation, execution, RTCS scheduling, cleaning, and help targets.

## Status Display and Program Control

The program uses the curses library to display the current action and sensor values. The keyboard is configured for non-blocking input, allowing the program to continue reading sensors while checking for a key press. Pressing 'q' exits the control loop. The motors are then stopped and the Initio and curses environments are cleaned up.

# Section 5: Build and Run

The supplied Makefile provides the following targets:

- make – compile the line_follower program.

- make run – compile and run the program.

- make schedule – run the program using rtcs_schedule.

- make clean – remove the compiled binary.

- make help – display the available commands.

The compiler flags include -Wall, -Werror, and -mfloat-abi=hard. The program links against the Initio, curses, wiringPi, and pthread libraries.